# Quartz liner set 1: 2020 9 October - 31 December

Visualization of the flame of  thescaled Flamesheet combustor with quartz liner

IMPORTANT NOTES:

- H_gap is assumed to be 1.5 mm throughout all experiments

- SCHUIFMAAT 1 was used to measure dimensions and distances

# 9 October 2020

# 14 October 2020

# 5 November 2020

# 10 November 2020

H2% = 0

Phi = 0.90

Liner: Quartz

Test 1

Notes

- Hm* = 141.3 mm (gemeten ter hoogte van mixer 6)

- Video cam0:

  - Afgekapt door insufficient free physical memory

  - Overbelicht -> stalen probe niet goed zichtbaar

-----------------------------------------------------------------------------------------------------------------

H2% = 0

Phi = 0.90

Quartz

Test "fooling around"

Notes

- Hm* = 141.3 mm (gemeten ter hoogte van mixer 6)

- Geen video cam0 -> belichting checken met NI MAX

# 11 November 2020

H2% = 50

Phi = 0.70

Liner: Quartz

Test "fooling around"

Notes

- IMPORTANT: WAYS TO SEE WHERE THE FLAME ANCHORS

# 12 November 2020

H2% = 40

Phi = 0.70

Liner: Quartz

Test "fooling around"

Notes

- Demo to Ivan Langella and Georg Eitelberg from the Aerospace Department (LR):

  - Discussion regarding position/ location of the flame

# 25 November 2020

H2% = 75

Phi = 0.70

Liner: Quartz

Test 1

Goal: Finding limits of combustor design regarding operating conditions

Notes

- Hm* = 141.4 mm

- No thermocouple data (not turned on -> mistake)

# 27 November 2020

H2% = 50

Phi = 0.70

Liner: Quartz

Test 9

Goal: Finding correct height of liner for flashback map

Notes

- Hm* = 141.2 mm

- uFB ≈2.7 ms-1

- Flame detection works during normal operation, but not during flashback (still visible on the other side of the quartz glass liner).

- Gap height not measured (H_gap)

H2% = 50

Phi = 0.70

Liner: Quartz

Test 10

Goal: Finding correct height of liner for flashback map

Notes

- Hm* = 142.5 mm

- Flashback occurred at higher velocity than test 9

- Gap height not measured (H_gap)

H2% = 50

Phi = 0.70

Liner: Quartz

Test 11

Goal: Finding correct height of liner for flashback map

Notes

- Hm* = 140.6 mm

- Gap height not measured (H_gap)

H2% = 50

Phi = 0.70

Liner: Quartz

Test 12

Goal: Finding correct height of liner for flashback map

Notes

- Hm* = 140.0 mm

- Gap height not measured (H_gap)

- Cam0 video not complete due to Matlab memory error (!)

- Flashback velocity lowest of all test (9, 10, 11), but restricted operating range

# 8 December 2020

H2% = 50

Phi = 0.50

Liner: Quartz

Test 1

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- Check data cam0 and mfc

-----------------------------------------------------------------------------------------------------------------

H2% = 50

Phi = 0.60

Liner: Quartz

Test 1

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

-----------------------------------------------------------------------------------------------------------------

H2% = 50

Phi = 0.70

Liner: Quartz

Test 13

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

-----------------------------------------------------------------------------------------------------------------

H2% = 50

Phi = 0.80

Liner: Quartz

Test 1

Notes

- Hm* = 141.45 mm (gemeten ter hoogte van mixer 6)

-----------------------------------------------------------------------------------------------------------------

H2% = 50

Phi = 0.90

Liner: Quartz

Test 1

Notes

- Hm* = 141.25mm (gemeten ter hoogte van mixer 6)

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.35

Liner: Quartz

Test 1

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- Flame not detected by UV sensor -> bypass UV sensor?

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.40

Liner: Quartz

Test 1

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- Flame not detected by UV sensor -> bypass UV sensor?

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.50

Liner: Quartz

Test 1

Notes

- Hm* = 141.15 mm (gemeten ter hoogte van mixer 6)

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.60

Liner: Quartz

Test 2

Notes

- Hm* = 141.3 mm (gemeten ter hoogte van mixer 6)

- Flashback bij toenemende u_u ?!

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.60

Liner: Quartz

Test 3

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- Flashback door opwarmen keerkom

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.60

Liner: Quartz

Test 4

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.70

Liner: Quartz

Test 2

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.70

Liner: Quartz

Test 3

Notes

- Hm* = 141.3 mm (gemeten ter hoogte van mixer 6)

- Set parameters not achieved!

# 9 December 2020

H2% = 75

Phi = 0.70

Liner: Quartz

Test 4

Notes

- Hm* = 141.10 mm (gemeten ter hoogte van mixer 6)

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 75

Phi = 0.70

Liner: Quartz

Test 5

Notes

- Hm* = 141.15 mm (gemeten ter hoogte van mixer 6)

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.30

Liner: Quartz

Test 1

Notes

- Hm* = 141.15 mm (gemeten ter hoogte van mixer 6)

- Flame not detected -> UV sensor relocated

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.30

Liner: Quartz

Test 2

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- uFB ≈2.375 ms-1

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.35

Liner: Quartz

Test 1

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- uFB ≈3.900 ms-1

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.40

Liner: Quartz

Test 1

Notes

- Hm* = 141.15 mm (gemeten ter hoogte van mixer 6)

- Resonance -> "organ pipe"

- uFB ≈6.500 ms-1

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.45

Liner: Quartz

Test 1

Notes

- Hm* = 141.2 mm (gemeten ter hoogte van mixer 6)

- u ≈9.250 ms-1-> Velocity to high?

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.45

Liner: Quartz

Test 2

Notes

- Hm* = 141.7 mm [!] (gemeten ter hoogte van mixer 6)

- Velocity to low?

- Set parameters not achieved!

# 10 December 2020

H2% = 100

Phi = 0.45

Liner: Quartz

Test 3

Notes

- Hm* = 141.25 mm (gemeten ter hoogte van mixer 6)

- Velocity to low?

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.45

Liner: Quartz

Test 4

Notes

- Hm* = 142.2 mm [!] (gemeten ter hoogte van mixer 6)

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.45

Liner: Quartz

Test 5

Notes

- Hm* = 141.1 mm (gemeten ter hoogte van mixer 6)

- Hm* = 141.5 mm (gemeten ter hoogte van mixer 2)

- Set parameters not achieved!

-----------------------------------------------------------------------------------------------------------------

H2% = 100

Phi = 0.45

Liner: Quartz

Test 6

Notes

- Hm* = 140.9 mm (gemeten ter hoogte van mixer 6)

- Set parameters not achieved!

# 15 December 2020

H2% = 100

Phi = 0.45

Liner: Quartz

Test 7

Metalen opzetstuk op liner: Thermo-acoustische verschijnselen onderdrukken

Notes

- Hm* niet gemeten (kan niet door metalen opzetstuk)

- Set parameters not achieved!

- Metalen opzetstuk levert niet het gewenste effect.

# 16 December 2020

| Test case | Timer labview (first frame) | Trigger time labview cam1 txt file (start time) | Trigger time python cam0 txt file (start time) | Difference Labview timer and camX [s] | Average 1 [s] | Difference cam0 and cam1 (average) [s] | Files |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NIKON 1 | 13:58:02.278 | 13:58:02.130 | - | 0.148 | 0.16033 | 1.20233 | sync_test_2020-12-16_test1_cam1 |
| NIKON 2 | 14:23:04.977 | 14:23:04.794 | - | 0.183 | 0.16033 | 1.20233 | sync_test_2020-12-16_test2_cam1 |
| NIKON 3 | 14:26:17.384 | 14:26:17.234 | - | 0.150 | 0.16033 | 1.20233 | sync_test_2020-12-16_test3_cam1 |
| ELP 1 | 14:11:23.928 | - | 14:11:25.059 | -1.131 | -1.042 | 1.20233 | sync_test_2020-12-16_test1_cam0 |
| ELP 2 | 14:16:47.673 | - | 14:16:48.684 | -1.011 | -1.042 | 1.20233 | sync_test_2020-12-16_test2_cam0 |
| ELP 3 | 14:21:39.189 | - | 14:21:40.173 | -0.984 | -1.042 | 1.20233 | sync_test_2020-12-16_test3_cam0 |